# ME4640 ROBOT HOMEWORK B

Spring 2026

# Learning Objectives:

In this assignment, you will:

- Manually control your 4-DOF robot using the HRIB joysticks

- Establish a consistent home configuration and determine servo angle offsets

- Command smooth joint trajectories programmatically

- Generate and verify joint angle plots before executing motion

General Note: Make sure to only run the robot continuously for a few minutes at a time and remove power from it when it’s not in use (hobby servos are not designed for continuous operation and can overheat).

# Part 1:  Create a Joystick Control Program with Filtering and Deadband

In this part, you will write an Arduino code that allows you to control all four robot joints using the two joysticks on the HRIB shield.

You will use this program again in Part 2 to find the robot’s home calibration offsets.

## Hardware Setup

  - Mount the HRIB shield onto your Arduino Uno and connect the four servos:

| - Joint | - Servo | - Pin |
| --- | --- | --- |
| - Base | - Servo 1 | - D3 |
| - Shoulder | - Servo 2 | - D5 |
| - Elbow | - Servo 3 | - D6 |
| - Gripper | - Servo 4 | - D9 |

  - In your Arduino program, you will map the Joystick inputs as follows:

| - Axis | - Pin | - Joint |
| --- | --- | --- |
| - Base | - A2 | - Base |
| - Shoulder | - A3 | - Shoulder |
| - Elbow | - A0 | - Elbow |
| - Gripper | - A1 | - Gripper |

  - Use analogRead()  to read each axis. You will read values between  0 and 1023, resting near 512 when the joystick is centered.

- Apply a deadband

The joystick readings may read offset values at the center (due to hysteresis effects and manufacturing differences), which can cause unwanted motion of the robot. You will add a deadband around the joystick’s center values (~512) so that the robot only moves when desired.

Example code:

If(abs(J1 – 512) < 20)

J1 = 512;

- Filter the Joysticks

The joystick potentiometers are noisy. If you directly applied their output to command the servos in analogRead(), the robot will jitter.

You will use an IIR (Infinite Impulse Response) low-pass filter, which behaves similarly to a moving average (but is more computationally efficient).

The filtered angle position update can be implemented as:

J1_new = (1-alpha)*J1_old + alpha*J1_deadbanded; // apply IIR filter to analog input

th1 = th1 + K1*J1_new;  //  update th1 position

Where:

- α is between 0 and 1 ( a value between 0.05 and 0.15 is a good starting range)

- Small α→ heavy smoothing (slower response)

- Large α→ less smoothing (slower response)

- K1 is a proportional gain to adjust the joystick sensitivity

Tune your choice for α to achieve smooth, stable control of each joint.

### Program Requirements

Your code must:

- Read all four joystick axes.

- Apply IIR filtering to each.

- Apply a deadband around the center.

- Map filtered values (0-1023) to updated joint angle positions

  - Implement software limits on the ROM for each joint (hint: use constrain())

  - To avoid damage, do not command servos to drive into “hardstops” or run them continuously for more than a few minutes at a time.

- Output to the Serial Monitor:

  - θ1-4 is servo microseconds and degrees. Use your calibrations from HW A 
(which you will modify in part 2 to apply home pose offsets)

## Part 2: Determine Home Offsets

Using your joystick control program:

- Move your robot to the defined home position (see Figure 1).

- Record the microsecond value for each servo at home.

- These are your calibration offsets (in µs).

In the home configuration we want: θ1=θ2=θ3=θ4=0°. Apply your servo offsets to adjust your calibration equations (from HW A)

Figure 1. Home Position for the MeArm-v3.0 In the home position, the robot should point straight up, then straight up, and then straight out again.

### Video Demonstration of Joystick Mode

Record a short video (30-60 sec):

- Demonstrate control of all four joints

- Show smooth, responsive control

- Show that in the home position, the joint positions in degrees printed to Serial are 0°

  - You may wish to use one of the buttons to reset the command position to home.

    - Avoid large step inputs! Make motions smooth! (This might be easier after finishing part 3)

- State the assignment number and your name during the video or include a title card.

## Part 3: Programmed Joint Trajectories

Now replace the joystick control mode with a programmed trajectory using the guidelines below.

Create an Arduino program that automatically moves your robot.

- All motions must be smooth. No step changes to angles.

- Choose your own target angles:

  - Choose angles that make your robot's motion clear in your video.

Your code must:

- Start at home.

- Sweep at constant angular velocity from  to over 2 seconds.

- Hold at  for 1 second.

- Sweep at constant angular velocity from  to over 2 seconds.

- Hold at  for 1 second.

- Sweep at constant angular velocity from  to over 2 seconds.

- Hold at  for 1 second.

- Open the gripper at constant angular velocity over 1 second.

- Close the gripper at constant angular velocity over 1 second.

- Sweep at constant angular velocity from  to over 2 seconds.

Required Verification Plot (Before Running on Robot)

Before using your code with powered servos:

- Comment out your servo.writeMicroseconds()statements and run the code to confirm SerialMonitor outputs.

- Output:

  - Time

  - θ1-θ4 in degrees

- Plot θ1-θ4 vs. time on a single figure in MATLAB or Python.

  - Your plot should clearly show ramps, holds, and gripper motion.

- Do not skip this step. Many mechanical failures come from skipping verification.

Video Demonstration of Trajectory Mode

Record a short video showing:

- The full programmed motion sequence with smooth motions

- Your name and assignment # (spoken during the video or via text added to the video).

# WHAT TO SUBMIT

Complete the above and submit the following files (do not zip!) on Canvas:

- RobotB_Lastname_TrajectoryPlot.pdf (or .png)

- Single document containing:

- One clean plot of θ1-θ4 vs time

- Proper axis labels, legend, and title

- Font size of 12 pts or larger

- RobotB_Lastname_video.mp4 (or .mov)

- One stitched together video file demonstrating:

  - Joystick control (Part 2)

  - Programmed trajectory (Part 3)

- RobotB_Lastname_Arduino.pdf:

Your final Arduino code:/codes

- Comment your full name at the top

- Include both:

  - Joystick mode

  - Trajectory mode

- Keep IDE formatting (Colored syntax!)

- RobotB_Lastname_PlotCode.pdf

- Full name comment at top

- Keep IDE formatting (Colored code syntax!)

- Include all code required to generate your trajectory verification plot.

GRADING RUBRIC

- Part 1: Joystick control with filtering, deadband, and ROM limits (25%)

- Part 2: Home calibration offsets and joystick demonstration video (20%)

- Part 3: Programmed joint trajectories and smooth execution (25%)

- Trajectory verification plot (θ₁–θ₄ vs. time) (15%)

- Code quality, organization, and documentation (10%)

- Proper file naming and submission format (5%)